1    GENERAL

This section relates to the supply and installation of CS FOR DOORS® specialised cavity sliders and doors.

It includes:

- Ultimate®

- EasyOpen® WC

- ElecTrack®

- X-RayBarrier

- SoundStop®

- BraceWall®

- Hi-ImpactJambs®

- Ultimate® Heavy Duty

- Specialised Door Leaves

- Automated Doors

- Track Systems

- Pre-hung aluminium jambs

1.1    RELATED WORK

Refer to glazing section/s for glazing for timber doors

Refer to electrical section/s for electrical supply and connections to Automatic Doors

Documents

1.2    DOCUMENTS

Refer to the general section 1233 REFERENCED DOCUMENTS. The following documents are specifically referred to in this section:

| AS/NZS 1170.1 | Structural design actions - Permanent, imposed and other actions |
| --- | --- |
| NZS 1170.5 | Structural design actions - Earthquake actions - New Zealand |
| NZS 3602 | Timber and wood-based products for use in building |
| NZS 3610 | Specification for profiles of mouldings and joinery |
| NZS 4121 | Design for access and mobility - Buildings and associated facilities |
| NZS 4223.3 | Glazing in buildings - Human impact safety requirements |

1.3    MANUFACTURER/SUPPLIER DOCUMENTS

Manufacturer’s and supplier’s documents relating to this part of the work:

CS FOR DOORS® Specifiers Guide

BIA Accreditation #93/006A

BRANZ Appraisal 900 - CS Cavity Sliders

Manufacturer/supplier contact details

| Company: | CS FOR DOORS® |
| --- | --- |
| Web: | www.csfordoors.co.nz |
| Email: | info@csfordoors.co.nz |
| Telephone: | 09 276 0800 Auckland |
|  | 021 630 800 Auckland |
|  | 07 928 0800 Waikato/Bay of Plenty |
|  | 04 473 9994 Wellington |
|  | 03 348 6158 South Island |

Warranties

1.4    WARRANTY

Provide the following CS FOR DOORS® warranties:

10 Years on the following product categories:

Cavity Sliders, Wardrobe Sliders, Pre- Hung Jambs, Track Systems.

5 Years on the following product categories:

CS manufactured Door Leaves, Automatic Units

2 Years on the following product categories:

CaviLock® Handle Hardware

12 Months on the following product categories:

Electrical components and parts

Provide this warranty on the manufacturer's standard form.

- Refer to the general section 1237 WARRANTIES for additional requirements. Refer to www.csfordoors.co.nz for guarantee terms and conditions.

Requirements

1.5    NO SUBSTITUTIONS

Substitutions are not permitted to any specified system, or associated components and products.

1.6    QUALIFICATIONS

Carry out the installation work with trained, experienced, competent installers familiar with the products being used.

2    PRODUCTS

Materials - general

2.1    TIMBER

Solid timber to NZS 3602 to profiles detailed.  Moisture content approximately 16% ex factory.

2.2    GLASS

Refer to glazing section/s for glazing to timber and aluminium doors.

2.3    INTERIOR TIMBER

To NZS 3602.  Moisture content approximately 10-14%.

Materials - doorsets

2.4    SPECIALISED CAVITY SLIDER UNITS

CS CavitySliders® doors to profile as scheduled, detailed and dimensioned.  Refer to SELECTIONS for options.

2.5    DISABLED ACCESS TOILET CAVITY SLIDER UNITS

CS CavitySliders® doors to profile as scheduled, detailed and dimensioned to NZS 4121. Refer to SELECTIONS for options.

2.6    STANDARD DOORSETS, SLIDING, HINGED, PIVOTING

Frames to profile as detailed and dimensioned.

Components

2.7    DOOR FURNITURE

CS CaviLock® architectural handles and locks as scheduled.  Refer to SELECTIONS for options.

2.8    SCREWS

Stainless steel or non-corrodible metal.  Length sufficient to penetrate into the background support up to the shank.  Screws for fixing hinges, hardware or furniture to match the item being attached.

2.9    NAILS

Length sufficient to penetrate into the background support at least half the nail length, except if into radiata pine then three-fifths their length.

2.10   SLIDING DOOR GEAR

CS CavitySliders® track to suit door size, weight and application.  Stainless steel carriages and mount plates are available for the CS240 kg and CS500 kg systems.

Finish

2.11   FINISH

~

3    EXECUTION

Conditions

3.1    GENERALLY

Execution to include those methods, practices and processes contained in the unit standards for the National Certificate in Carpentry and the National Certificate in Joinery (cabinetry, exterior joinery, stairs).

3.2    DO NOT DELIVER

Do not deliver any elements which cannot be unloaded immediately into suitable storage conditions.

3.3    HANDLE

Handle, unload and store elements without distortion and avoiding pre-finished surfaces rubbing together, and contact with mud, moisture and other damaging materials.

3.4    PROTECT

Protect all elements against damage to arrises and glazing beads.  Store frames and doors flat and away from moisture or direct sunlight.

3.5    FABRICATE DOORSETS

Fabricate doorsets in the CS FOR DOORS® factory with doors hung, provision for furniture made, finishes applied and fully operable.

3.6    FABRICATE DOORS

Fabricate doors in the factory, with provision for door furniture.

3.7    CHECK ALL OPENINGS

Check all openings on site for size and standard of execution before installing window or door frames.  Installation tolerances of windows subject to earthquake design to comply with AS/NZS 1170.1 or NZS 1170.5.

Assembly

3.8    FABRICATION GENERALLY

Manufacture and fabricate frames and doors as detailed.  Install running gear as scheduled.  Provide temporary bracing and protection.  Temporarily secure all opening elements for transportation.

Application - generally

3.9    FIXING FRAMES

Fit flashings to frame and framing as required.  Fix and assemble frames rigidly in place, plumb, level and true to line and face without distortion.  Fit facings, scribers, draught-stopping and sealants.

3.10   DISTORTION

Do not distort frames when wedging or other packing, or when tightening fixings.  If necessary adjust packing and fixings to eliminate binding.  Do not cut, plane or sand frames to remedy distortion.

3.11   FIXINGS

Fix frames so that nail heads are covered by applied stops and beads.  Punch all nail heads below timber surfaces which will be visible in completed work.  Ensure that at least one frame fixing is adjacent to each hanging point.

Application - doorsets

3.12   PROPRIETARY ELEMENTS

Fix in accordance with CS FOR DOORS® requirements.

3.13   INSTALLATION GENERALLY

Frames finished to match the width of lined walls. Wedge frames into opening and nail through into the studs making sure you have one fixing in the vicinity of the lock.

Hang doors on sliding gear as specified and to operate freely. Fit all hardware and door furniture to CS FOR DOORS® instructions.

3.14   TIMBER STUD WALLS - TIMBER FRAMES

Fix direct to opening and pack, with one fixing in the vicinity of the lock.

3.15   STEEL STUD WALLS - STEEL FRAMES

Using a pilot hole in the frame, fix to steel studs with countersunk self-drilling corrosion proof screws. Fix direct to opening and pack, with one fixing in the vicinity of the lock.

3.16   STEEL STUD WALLS - TIMBER FRAMES

Drill the timber frame and fix to steel studs with countersunk self-drilling corrosion proof screws. Fix direct to opening and pack, with one fixing in the vicinity of the lock.

3.17   BOTTOM CLEARANCE

Provide for specified floor coverings plus a minimum of 5mm clearance at any point of slide. When floor covering is not specified, allow 25mm total.

For ventilated and/or air conditioned spaces allow 20mm clearance above finished floor coverings for supply/return air.

3.18   REMOVE DOORS

Remove doors from the frames if necessary to protect them, or for re-finishing, store safely and near completion refit them, all without any damage.

3.19   INSTALL PANELS

Prime rebates and beads, install sealant backing strips or silicone.  Install dry beading to outside of panels as selected.  Do not mitre corners of beads.

3.20   MANIFESTATIONS

To NZS 4223.3, Manifestation (making glass visible).

3.21   INSTALL FURNITURE

Install latches, locks and door furniture as scheduled.

3.22   CHECK

Check and adjust operation of all doors, hardware and furniture.

Completion

3.23   PROTECTION

Protect all finishes against damage from adjacent and following work.

3.24   REPLACE

Replace damaged, cracked or marked elements.

3.25   TRADE CLEAN

Clean off or remove safety indicators at completion of the building.

3.26   LEAVE

Leave work to the standard required for following procedures.

3.27   REMOVE

Remove safety indicators and protective coverings, and wipe down all doorsets thoroughly to leave them perfectly clean.  Remove all debris, unused materials and elements from the site.

4    SELECTIONS

For further details on selections go to www.csfordoors.co.nz

Substitutions are not permitted to the following, unless stated otherwise.

CS Cavity Sliders® Specialised Range

4.2    CS CAVITYSLIDERS® EASYOPEN® COMPLETE DISABLED ACCESS TOILET DOORS

| Location: | ~ |
| --- | --- |
| Leaf dimensions: | ~mm x 910mm minimum |
| Door type: | ~ |
| Door finish: | ~ |
| Single or Biparting: | Single only |
| Jamb type/finish: | ~ |
| Framing/size: | ~mm |
| Lining thickness: | ~mm |
| Track finish: | ~ |
| Handle type: | CL 100 LaviLock® by CaviLock® |